Interrupts in Mars

There are some differences between how interrupts can be used in Mars compared to the slides. This is because the slides describe the MIPS (R2000) architecture while Mars simulates the MIPS32 architecture.

First the interrupt routine address in Mars is by default 0x80000180 rather than 0x80000080.

Second, Mars has a very strict memory control regarding kernel memory and self-modifying code. Therefore, we cannot have code that copies other code into the kernel area of the memory. So, in order to get the driver for the interrupt routine in place the following code needs to be used:

.ktext 0x80000180

la $t1, int_routine

jr $t1

nop

Thirdly, the command for exiting interrupt is rfe in R2000 and eret in MIPS32. The difference is that the eret command will put the value in the EPC-register into the PC-register automatically.

In all other important aspects interrupts work as expected. However, do not expect the coprocessor 0 register view to update in real-time it will only do that (for some reason) when you step through the program.

Finally: there are some places in the example code where the following pre-processor construct is used EXT_INT3 | MY_VALUE. This is not understood by the Mars environment but the meaning of the code is bitwise or between EXT_INT3 and MY_VALUE so there are other ways of handling it in Mars.

Instructions for using the traffic light tool.

The traffic light tool provides two traffic lights. One for car traffic and one for pedestrians.

The colour or mode of these lights are controlled through memory mapped I/O where the Byte value at address 0xFFFF0010 : controls the traffic light for the pedestrians:

Dark=0x00

Stop=0x01 (bit 0)

Walk=0x02 (bit 1)

Byte value at address 0xFFFF0011 : controls the normal traffic light with:

Dark=0x00

Red=0x01 (bit0)

Orange=0x02 (bit1)

Green=0x04 (bit2)

In order to light one of the colours in the traffic light the values above are written to the memory address e.g. if we want to have a red light the following code is used:

la      $t0, 0xFFFF0011   	#Set red light om main road

add   $t1, $zero, 0x01

sb      $t1, 0x0($t0)

The tool also has two push buttons and these generate interrupts on external interrupt 1 (i.e. Bit 11 in the status register) when pushed. Which button that has been pushed is stored in the memory byte at address 0xFFFF0013 where the walk button = 0x01 (bit 0) and the drive button = 0x02 (bit 1).  The value is overwritten the next time a button is pressed.

The tool also contains a timer. The timer is enabled by writing any value (> 0) to address 0xFFFF0012.  If timer interruption is enabled, every 30 instructions, the timer generates interrupts on external interrupt 0 (i.e. Bit 10 in the Status register) this can be used as one time unit for labs etc.

(PLEASE DO NOT USE THE TIMER WHEN MARS IS RUNNING AT FULL SPEED ALWAYS HAVE THE SPEED SET TO < 30 inst./Sec.)